Network Documentation - Modules 11-13

# IPv4 Subnet Scheme (VLSM)

Address Space: 192.168.1.0 /24

Method: Variable Length Subnet Mask (VLSM) - sorted largest to smallest

| Subnet | Network Address | Subnet Mask | Prefix | First Host | Last Host | Broadcast | Need | Get |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LAN-A | 192.168.1.0 | 255.255.255.192 | /26 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 | 60 | 62 |
| LAN-B | 192.168.1.64 | 255.255.255.224 | /27 | 192.168.1.65 | 192.168.1.94 | 192.168.1.95 | 25 | 30 |
| LAN-C | 192.168.1.96 | 255.255.255.224 | /27 | 192.168.1.97 | 192.168.1.126 | 192.168.1.127 | 20 | 30 |
| LAN-D | 192.168.1.128 | 255.255.255.240 | /28 | 192.168.1.129 | 192.168.1.142 | 192.168.1.143 | 12 | 14 |
| LAN-E | 192.168.1.144 | 255.255.255.240 | /28 | 192.168.1.145 | 192.168.1.158 | 192.168.1.159 | 12 | 14 |
| P2P-1 | 192.168.1.160 | 255.255.255.252 | /30 | 192.168.1.161 | 192.168.1.162 | 192.168.1.163 | 2 | 2 |
| P2P-2 | 192.168.1.164 | 255.255.255.252 | /30 | 192.168.1.165 | 192.168.1.166 | 192.168.1.167 | 2 | 2 |
| P2P-3 | 192.168.1.168 | 255.255.255.252 | /30 | 192.168.1.169 | 192.168.1.170 | 192.168.1.171 | 2 | 2 |
| P2P-4 | 192.168.1.172 | 255.255.255.252 | /30 | 192.168.1.173 | 192.168.1.174 | 192.168.1.175 | 2 | 2 |

# IPv6 Routing Table (NET_1_R1)

Command used: show ipv6 route

| Type | Network | Prefix | Via |
| --- | --- | --- | --- |
| C | 2001:ABCD:1234:1::/64 | /64 | GigabitEthernet0/0/0, directly connected |
| L | 2001:ABCD:1234:1::1/128 | /128 | GigabitEthernet0/0/0, receive |
| C | 2001:ABCD:1234:2::/64 | /64 | GigabitEthernet0/0/1, directly connected |
| L | 2001:ABCD:1234:2::1/128 | /128 | GigabitEthernet0/0/1, receive |
| L | FF00::/8 | /8 | Null0, receive |

C = Connected (directly connected network)

L = Local (IP address of the router interface itself)

# Router Configuration (NET_1_R1)

Command used: show running-config

hostname NET_1_R1

!

ipv6 unicast-routing

!

interface GigabitEthernet0/0/0

description Link to Room 100 LAN

ip address 10.0.0.1 255.255.255.0

ipv6 address FE80::1 link-local

ipv6 address 2001:ABCD:1234:1::1/64

!

interface GigabitEthernet0/0/1

description Link to Admin and Wireless LAN

ip address 11.0.0.1 255.255.255.0

ipv6 address FE80::1 link-local

ipv6 address 2001:ABCD:1234:2::1/64

!

ip route 192.168.0.0 255.255.255.0 11.0.0.2

# IPv6 Address Configuration on PCs

| PC | IPv6 Address (GUA) | Prefix | Link-local | Default Gateway |
| --- | --- | --- | --- | --- |
| PC1 | 2001:ABCD:1234:1::15 | /64 | FE80::15 | FE80::1 (G0/0/0) |
| PC2 | 2001:ABCD:1234:1::16 | /64 | FE80::16 | FE80::1 (G0/0/0) |
| PC3 | 2001:ABCD:1234:1::17 | /64 | FE80::17 | FE80::1 (G0/0/0) |
| PC4 | 2001:ABCD:1234:1::18 | /64 | FE80::18 | FE80::1 (G0/0/0) |
| PC5 | 2001:ABCD:1234:1::19 | /64 | FE80::19 | FE80::1 (G0/0/0) |
| Admin1 | 2001:ABCD:1234:2::15 | /64 | FE80::15 | FE80::1 (G0/0/1) |
| Admin2 | 2001:ABCD:1234:2::16 | /64 | FE80::16 | FE80::1 (G0/0/1) |